Standard Operating Procedure

Laboratory Specific

Chemical: Phenol

Please fill out the form completely.  Print a copy and insert into your

Laboratory Safety Manual and Chemical Hygiene Plan.

Refer to instructions for assistance.

__

| Department: | Chemistry & Biochemistry |
| --- | --- |
| Date SOP was written: | 1/15/13 |
| Date SOP was approved by PI/lab supervisor: | 1/15/13 |
| Principal Investigator: | Irene Chen |
| Internal Lab Safety Coordinator/Lab Manager: | Irene Chen |
| Lab Phone: | 805-893-8085 |
| Office Phone: | 805-893-8364 |
| Emergency Contact: | Irene Chen, 617-710-8741 (cell) |
|  | (Name and Phone Number) |
| Location(s) covered by this SOP: | Chemistry 1142 |
|  | (Building/Room Number) |

Type of SOP:            Process            Hazardous Chemical             Hazardous Class

Purpose

It is only mildly acidic but requires careful handling due to its toxicity and its propensity to cause severe burns.

Physical & Chemical Properties/Definition of Chemical Group

Phenol, also known as carbolic acid, is an organic compound with the chemical formula C6H5OH. It is a white, crystalline solid at room temperature. It consists of a phenyl (-C6H5) group, bonded to a hydroxyl (-OH) group.

Phenol is commonly used in molecular biology techniques such as to separate proteins from nucleic acids, usually in combination with chloroform. It is a corrosive that affects the central nervous system, liver and kidneys. Phenol is irritating to the skin but has a local anesthetic effect. Hence, no pain may be felt on initial contact, but severe burns may develop after sometime. Phenol is rapidly absorbed through the skin so toxic amounts may be absorbed through relatively small areas.

Synonyms:		Hydroxybenzene

Carbolic acid

Benzenol

Phenylic acid

Phenic acid

Phenyl alcohol

CAS#:      		108-95-2

Molecular formula:      	C6H6O

Molecular weight:          	94.11 g

Form: 		white crystalline solid

Phenol is appreciably soluble in water.

Potential Hazards/Toxicity

Phenol and its vapors are corrosive and hazardous to the eyes, the skin, and the respiratory tract. Repeated or prolonged skin contact with phenol may cause dermatitis, or even second and third-degree burns due to phenol's caustic and defatting properties. Inhalation of phenol vapor may cause lung edema. The substance may cause harmful effects on the central nervous system and heart, resulting in dysrhythmia, seizures, and coma. The kidneys may be affected as well. Exposure may result in death and the effects may be delayed. Long-term or repeated exposure of the substance may have harmful effects on the liver and kidneys.

Toxic Effects:	Phenol is a toxic substance. Phenol is a poison and toxic via ingestion, inhalation and skin absorption. This chemical is easily absorbed through the skin and fatalities have been documented from skin absorption through a relatively small surface area.

Acute Effects:	Acute phenol intoxication causes shock, collapse, coma, convulsions, cyanosis, and death. Ingestion of lethal amounts causes severe burns of the mouth and throat, marked abdominal pain, cyanosis, muscular weakness, collapse, coma, and death. Tremors, convulsions, and muscle twitching have also occurred. Contact of the skin with the solid or liquid can produce chemical burns, redness, edema, tissue necrosis, and gangrene; contact with the eye may result in irritation, conjunctival swelling, whitened cornea, and blindness.

Chronic Effects:            	Chronic phenol poisoning is characterized by vomiting, difficult swallowing, excessive salivation, diarrhea, anorexia, headache, fainting, vertigo, mental disturbances, and possibly skin eruptions. Prolonged cutaneous exposure may result in deposition of dark pigment in the skin.

Carcinogenic Effects:    	Not considered carcinogenic

Reproductive Hazard:	            May cause reproductive and fetal effects.

Exposure Limits

The current OSHA permissible exposure limit (PEL) for phenol is 5 ppm (19 mg/m3) as an 8-hour time-weighted average (TWA) concentration. The OSHA PEL also bears a “Skin” notation, which indicates that the cutaneous route of exposure (including mucous membranes and eyes) contribute to the overall exposure.

The NIOSH established a recommended exposure limit (REL) for phenol of 5 ppm (19 mg/m3) as a TWA for up to a 10-hour workday and a 40-hour work week.  The short-term exposure limit (STEL) for phenol is 15.6 ppm for periods not exceeding 15 minutes. NIOSH also bears the “Skin” notation to phenol.

The American Conference of Governmental Industrial Hygienists (ACGIH) assigned phenol a threshold limit value (TLV) of 5 ppm (19 mg/m3) as a TWA for a normal 8-hour workday and a 40-hour workweek.  The ACGIH also bears a “Skin” notation to phenol.

A worker’s exposure to phenol can be determined by analyzing a urine sample taken at the end of the shift for total phenol. A 250 mg total phenol per gram creatinine level corresponds to an airborne phenol exposure at the TLV (5 ppm). Dermal absorption of phenol may contribute to urinary levels found.

Route

Phenol can enter the system by skin absorption/dermal contact, inhalation or ingestion or through  eye contact.

Personal Protective Equipment (PPE)

- Neoprene (double glove), viton or vinyl gloves are recommended when handling phenol. Change gloves after handling phenol. Note: Nitrile gloves are a poor choice when working with phenol.

- Safety glasses with side shields or chemical splash goggles shall be worn.

- A chemical resistant laboratory coat should be worn when working with phenol.

- Closed toe shoes are required when working in the laboratory.

- If there is a potential for body splash, wear a rubber or neoprene apron.

Engineering Controls

- Work with phenol in a properly operating and certified chemical fume hood.

- Work at least 6” inside the hood, never place your head in the hood, set the sash at the lowest position possible (if using the horizontal sliding sashes do not open past the green tape mark).

First Aid Procedures

Chemical burns from skin exposures can be decontaminated by washing with polyethylene glycol, isopropyl alcohol or perhaps even copious amounts of water. Removal of contaminated clothing is required, as well as immediate hospital treatment for large splashes. This is particularly important if the phenol is mixed with chloroform (a commonly-used mixture in molecular biology for DNA & RNA purification from proteins).

- In the event of skin contact, immediately wash with soap or mild detergent and large amounts of water until no evidence of chemical remains (15-20 minutes).  Remove contaminated clothing and shoes immediately if necessary – recommend discarding contaminated clothing.  Seek medical attention.

- Olive oil and vegetable oil are effective in removing phenol from skin and retarding absorption.

- In case of contact with eyes, immediately flush eyes with copious amounts of water for at least 15 minutes (lifting upper and lower eyelids occasionally) and obtain medical attention.

- If inhaled, move to fresh air immediately and seek medical attention.

- In the event of ingestion, seek medical attention immediately.

- Report all incidents or near misses to UCLA EH&S at x59797and complete the appropriate Incident Forms.

Special Handling and Storage Requirements

- Keep container tightly closed to prevent agent from subliming and entering the atmosphere.

- Store away from strong oxidizers such as hydrogen peroxide, chlorine, bromine, acids, and calcium hypochlorite.

- Store saturated phenol at 4o C in designated secondary containers.

- Wash hands thoroughly after handling (even if gloves were used).

- Keep away from heat, sparks, flames, sources of ignition (including empty containers that held phenol, as the containers will have phenol residue).

- Store protected from light and moisture.

- Use only in a chemical fume hood.

- Transport chemicals in closed containers, in the smallest amounts possible, and use aids such as carts, chemical transport carriers, etc.

- It is highly recommended that all chemicals be stored below eye level so cracking or leaking containers are immediately visible and there is less potential for chemicals falling onto lab workers when pulling from shelves.

Spill and Accident Procedure

Note: Do not attempt cleanup if you feel unsure of your ability to do so or if you do not know the proper procedure to clean up the spill.

Small spills: Use vermiculite or commercially available spill absorbent material. Collect the spilled material in a clearly labeled hazardous waste container. Place absorbent pillows/pads around and flood area with water then cover with caustic soda ash to neutralize the residual material.

Large spills: Notify others in room. Evacuate the lab/room immediately. Call 911 and EH&S at x59797. Close lab doors and post warning signs at entrances/exits notifying others regarding the spill. Prevent unnecessary entry into the spill area. Provide assistance and information to spill responders.

Chemical Spill Dial 911 and call EH&S at 893-3194

Spill – Help contaminated or injured persons.  Evacuate the spill area.  Avoid breathing vapors.  Eliminate sources of ignition if the chemical is flammable.  If possible, confine the spill to a small area using a spill kit or absorbent material. Keep others from entering contaminated area (e.g., use caution tape, barriers, etc.).

Small (<1 L) – If you have training, you may assist in the clean-up effort.  Use appropriate personal protective equipment and clean-up material for chemical spilled.  Double bag spill waste in clear plastic bags, label and take to the next chemical waste pick-up.

Large (>1 L) – Dial 911 and EH&S at 893-3194 for assistance.

Chemical Spill on Body or Clothes – Remove clothing and rinse body thoroughly in emergency shower for at least 15 minutes.  Seek medical attention. Notify supervisor and EH&S at 893-3194 immediately.

Chemical Splash Into Eyes – Immediately rinse eyeball and inner surface of eyelid with water for 15 minutes by forcibly holding the eye open.  Seek medical attention. Notify supervisor and EH&S at 893-3194 immediately.

# Medical Emergency Dial 9-911

Life Threatening Emergency: Dial 9-911 Contact PI and EH&S as soon as practical. Note: All serious injuries must be reported to EH&S at x3194 within 8 hours.

Non-Life Threatening Emergency – Go to Student Health, building 588, x5361, Hours: M, T, R, F 8 a.m. to 4:30 p.m, W 9 a.m. to 4:30 pm and R 5 to 7 p.m. by appointment. At all other times report to Goleta Valley Cottage Hospital (emergency room) at 351 South Patterson Avenue, 805-967-3411. Contact PI and EH&S as soon as practical.Note: All serious injuries must be reported to EH&S at x3194 within 8 hours.

Decontamination/Waste Disposal Procedure

Wearing proper PPE, please decontaminate equipment and bench tops using water.  Please dispose of the used material and disposables contaminated with this material as hazardous waste.

General hazardous waste disposal guidelines:

Label Waste

- Affix a hazardous waste tag on all waste containers as soon as the first drop of waste is added to the container

Store Waste

- Store hazardous waste in closed containers, in secondary containment and in a designated location

- Double-bag dry waste using transparent bags

- Waste must be under the control of the person generating & disposing of it

Dispose of Waste

- Dispose of regularly generated chemical waste within 90 days

- Call EH&S for questions and for pick-up

- Empty Containers: Dispose as hazardous waste irrespective of the container size

Prepare for transport to pick-up location

- Check waste tag

- Use secondary containment

Safety Data Sheet (SDS) Location

Online SDS can be accessed at http://www.chem.ucsb.edu/about/safety .

Protocol/Procedure

We use phenol for DNA and RNA extraction, often in conjunction with chloroform.

NOTE: Any deviation from this SOP requires written approval from PI.

Documentation of Training (signature of all users is required)

- Prior to conducting any work with Phenol, designated personnel must provide training to his/her laboratory personnel specific to the hazards involved in working with this substance, work area decontamination, and emergency procedures.

- The Principal Investigator must provide his/her laboratory personnel with a copy of this SOP and a copy of the Phenol MSDS provided by the manufacturer.

- The Principal Investigator must ensure that his/her laboratory personnel have attended appropriate laboratory safety training or refresher training within the last two years.

I have read and understand the content of this SOP:

Name                                                       Signature                                      Date

________________________________		____________________________	____________

________________________________		____________________________	____________

________________________________		____________________________	____________

________________________________		____________________________	____________

________________________________		____________________________	____________

________________________________		____________________________	____________

________________________________		____________________________	____________

________________________________		____________________________	____________

________________________________		____________________________	____________

________________________________		____________________________	____________

________________________________		____________________________	____________

________________________________		____________________________	____________

________________________________		____________________________	____________

________________________________		____________________________	____________

________________________________		____________________________	____________

________________________________		____________________________	____________